| Date: | Customer: | Address: | Quote #: |
| --- | --- | --- | --- |
| 1/9/15 | Avani Environmental | Centrix, LLC | 010915-BP-Centrix |

| PROJECT SCOPE: |  |
| --- | --- |
| 12 gate automatic blast gate system for fire prevention |  |

| ANALYSIS: |  |
| --- | --- |
| Customer has requested for Absolent to provide a integrated control system for fire prevention. Customer’s CNC will provide it’s own signal to our logic control box. The logic control box will shut the gate for that CNC only, preventing draft inside CNC enclosure which would fuel fire. |  |

| RECOMMENDED SOLUTION: |  |
| --- | --- |
| Provide EcoBoss PowerBox integrated control panel with (12) EcoMaxx Automatic Blast Gates. Gates will be normally open. Signal from owner provided sensor will shut individual gate which will cut off airflow to individual CNC for fire protection. PowerBox control panel includes pressure transmitter for integration with optional EcoBoss or Absolent provided VFD. Pressure transmitter will speed up or slow down fan based on overall pressure drop based on filter loading at machine. This solution increases filter life and reduces fan noise. VFD priced separately. |  |

EcoBoss Energy Saving Control Upgrade (EEU):

| CONTROL PANEL(S): |  |  |
| --- | --- | --- |
|  | QTY. | DESCRIPTION |
|  | 1 | EB-PB-015.0-4803 ECOBOSS POWER BOX with the ability to operate 12 Autogates/Sensors, Internal User Friendly and Programmable Logic Controller, 0-10" WG Pressure Transducer, 24VDC Power Supply, Terminals for Connecting to SVD, VFD Link Light, 3-Prong 120VAC power cord, and terminals for connecting optional Fire Control unit, Fire Break/No Return |

| AUTO GATES: |  |  |
| --- | --- | --- |
|  | QTY. | DESCRIPTION |
|  | 12 | 5" ECOMAXX Automatic Blast Gate with Polyethyline Gasket, 24VDC Solenoid, LED Light and KB Ductwork Ends. Requires pneumatic connection to operate gate. |

| SENSORS: |  |  |
| --- | --- | --- |
|  | QTY. | DESCRIPTION |
|  |  | By Customer |
| CONTROL CABLE: |  |  |
|  | QTY. | DESCRIPTION |
|  |  | By Customer |

| PNEUMATICS |  |  |
| --- | --- | --- |
|  | QTY. | DESCRIPTION |
|  | 1 | By Customer |

| LABOR: |  |  |
| --- | --- | --- |
|  | QTY. | DESCRIPTION |
|  | 1 | By Customer |

| Price: | Additional Exclusions: | Notes: |
| --- | --- | --- |
| $ 7,280.70 | Freight, F.O.B. La Center, WA | Net Price reflects 30% distributor discount List Price: $ 10,401.00 |

Payment Terms:  40% deposit with order, 40% due when equipment is ready to ship, balance due upon completion of work and services provided by Boss Products, LLC.

Shipment:  3-4 weeks after receipt of written purchase order and deposit, sooner if possible.

Additional Items that are specifically not included:

- All shipping costs

- All labor

- All electrical connections

- All pneumatic connections

- Control Sensors (by customer)

- Electrical Disconnect boxes if required per code

- Permit fees –

- Any items that become a requirement during the course of the project that are not specifically stated above as included.

*Any change orders will be in writing and signed by both Boss Products, LLC and customer.  Terms of payment for any change orders will be net upon presentation of invoice.

**Freight charges are the responsibility of the customer and all shipments are to be sent freight collect unless otherwise discussed in advance of shipment.  Freight charges which are prepaid and added to the shipment are due immediately upon presentation of invoice and shall carry a handling charge.

Thank you for considering Boss Products, LLC for your energy saving control requirements.  We look forward to working with you on this project and others in the future!

Sincerely,

Boss Products, LLC

Chris Schlentz

**The above pricing information is strictly confidential.  As a trusted partner, we thank you for considering Boss Products, LLC for your ventilation requirements.  We look forward to working with you on this project and many others in the future!