Contractor Safety

Program

9728 Route 287

Wellsboro, PA 16901

(570)724-5191

Nov. 2009

TABLE OF CONTENTS		PAGE

I.	Introduction		4

A.	Purpose		4

II.	Ward/HMAC Responsibilities	4

A.	Administrative Duties		4

B.	Before contract work begins	4

C.	During contract work		5

III.	Contractor Responsibilities		5

A.	Contractor Duties		5

B.	During contract work, the contractor shall	6

IV.	Health, Safety and Environment (HSE)	6

A.	Health, Safety and Environmental Policy	6

B.	Compliance		6

C.	Unsafe Acts or Conditions		7

D.	HSE Training and Medical Evaluation	7

E.	HSE Performance Requirements	7

V.	Accident Prevention Programs		7

A.	Job Site Inspections		7

B.	Accident and Incident Investigations	7

C.	Safety Meetings		7

- Medical / First Aid Procedure		7

VII.	Housekeeping		8

- Personal Protective Equipment		8

A.	Eye/Face Protection		8

B.	Hearing Protection		8

C.	Hand Protection		8

D.	Fall Protection		8

E.	Foot Protection		8

F.	Clothing		9

G.	Head Protection		9

IX.	Tools		9

A.	Hand Tools		9

B.	Portable Power Tools		9

C.	Extension Cords		9

X.	Equipment		9

A.	General		9

B.	Ladders and Scaffolds		9

C.	Signs and Barricades		10

D.	Rigging		10

E.	Compressed Air		11

F.	Motor Vehicle and Power-Operated Equipment	11

- Fire Protection		11

A.	Alarm/Notification		11

B.	Flammable/Combustible Liquids	11

- Job Site Procedures		12

A.	Burning and Welding Procedures	12

B.	Hot Work Permit		12

C.	Confined Space Permit		12

D.	Electrical Work		12

E.	Lockout/Tagout Procedure		13

F.	Respirator Protection		13

G.	Hazardous Communication Program	13

H.	Excavations		14

I.	Steel Erections		14

J.	Demolition		14

K.	Insulation		14

L.	General Safety Rules		14

# I.	INTRODUCTION

Safety at Ward/HMAC is integrated into our manufacturing processes.  As a contractor coming onsite to perform work, safety must be part of your process also.

Ward/HMAC believe that all accidents are preventable and we strive to have a “ZERO ACCIDENT CULTURE”.  We are concerned about the safety of our employees and the employees of contractors performing work for Ward/HMAC facilities.  Merely talking about safety will not create a safe working environment.  Safety requires action.  Safety means performing your tasks in a safe manner and protecting people and equipment from hazards.

Working at Ward/HMAC in a safe manner is “A CONDITION OF EMPLOYMENT”.  You, as contractors, must assure that safe work practices are communicated and strictly adhered to by personnel.

# A.	PURPOSE

The purpose of this policy and procedure is to assure the safety and well being of all contractors hired to perform work, and the protection of the environment.  It is also to ensure that contractors comply with Ward/HMAC’s safety and environmental procedures, which promotes waste reduction, pollution prevention and conservation of energy and natural resources in accordance with the commitment stated in the Ward Quality and Environmental Policy or the HMAC Environmental Policy.

A written contractor safety policy establishes guidelines to be followed for contractors working at Ward/HMAC.  The rules established:

- Provide a safe working environment.

- Govern facility relationships with outside contractors.

- Ensure that contractor employees are trained to protect themselves from all potential and existing safety and environmental hazards.

The Contractor Safety Policy establishes uniform requirements designed to ensure that contractor safety orientation, coordination, and safety administration practices are communicated to and understood by contractor.

This document is provided to ensure all corporate safety plans, environmental policies and procedures are communicated to all participating contractors.  It also provides an avenue for contractors to communicate their safety plans, policies and procedures to Ward/HMAC.  This program aims to prevent personal injuries and illnesses, along with promoting protection of the environment.

# WARD / HMAC RESPONSIBILITIES

## A.	ADMINISTRATIVE DUTIES

The Health and Safety Manager shall be responsible for developing and maintaining the program. Contractor employees may review a copy of the plan.  It is located in the Health and Safety Office.  In addition, the Health and Safety Department is responsible for maintaining any records related to the contractor safety program.

- BEFORE CONTRACT WORK BEGINS:

- A representative from Health and Safety shall coordinate and communicate all safety and health issues with the contractor.

- Provide a copy of the facility's written safety policies and procedures to the contractor.

- Inform the contractor of any emergency signals and procedures that may be put into operation in areas where the contractor's employees are working.  The contractor should be given the telephone numbers of the nearest hospital, ambulance service, and fire department.

- Conduct an inspection of the proposed work-site area before the pre-start-up meeting so any known information about on-site hazards, particularly non-obvious hazards, are documented and thoroughly communicated to the contractor.

- Work directly with the contractor's designated representative, with whom all contacts should be made.

- Conduct a pre-start up meeting with the contractor's designated representative.

- Review all contract requirements related to safety and health with the contractor's designated representative, including, but not limited to, rules and procedures, personal protective equipment (PPE), and special work permits or specialized work procedures.  Advise the contractor that the facility safety and health policies must be followed.  A copy of the facility's safety plans must be furnished to the contractor.

- Inform contractor's designated representative of the required response to facility emergencies.

- Communicate thoroughly with the contractor's designated representative any safety and health hazards (particularly non-obvious hazards and hazard communication issues) known to be associated with the work, including those in areas adjacent to the work-site.

- Ensure that all affected employees at Ward/HMAC receive training on all hazards to which they will be introduced by a contractor.

- DURING CONTRACT WORK:

- Limit, as necessary, the entry of Ward/HMAC employees into contractor work areas.

- Monitor the contractor's compliance with the contract throughout the duration of the work.  When checking contractor work during the project, note any negligent or unlawful act or condition in violation of safety standards, environmental standards or other requirements.  Any items noted should be brought immediately to the attention of the contractor's designated representative in writing, with a copy of the notice being sent to the contractor's home office concurrently.  However, if an unsafe act or a condition is noted that creates an imminent danger of serious injury, immediate steps should be taken with the contractor's designated representative, or in his or her absence, the contractor's employees to stop the unsafe act or condition.  Do not allow work that is in violation of an OSHA, safety or environmental regulation to continue.

- Document all discussions, including place, time, date and names of contractor employees in attendance.

- For work for which Ward/HMAC have developed specific and generally applicable procedures, contractors and their subcontractors shall be required to follow the same procedures.

- Ward/HMAC’s policy is not to loan tools or equipment to outside contractors or their subcontractors.  The contractor is required to provide the necessary tools and equipment.

- Contact the nearest medical facilities, when available, in emergency situations where severity of the injury dictates immediate attention.

- After conclusion of the contract work, all relevant individuals shall complete a post-project assessment of the contractor's safety performance to be used for future reference, with a recommendation on whether or not to re-hire the contractor.

- CONTRACTOR RESPONSIBILITIES

A.	CONTRACTOR DUTIES

Contract employees must perform their work safely.  Considering that contractors often perform very specialized and potentially hazardous tasks, such as confined space entry activities and non-routine repair activities, their work must be controlled.  Contractor responsibilities when accepting contracts with Ward/HMAC include the following listed steps.  The contract employer will:

- Assure that all contract employees are trained in the work practices necessary to safely perform his or her job.

- Instruct the contract employee in the potential fire, explosion, or toxic release hazards related to his or her job and the process.

- Assure that the contract employee knows the applicable provisions of the emergency action plan.

- Document contract employee training.

- Inform contract employees of and then enforce safety rules of the facility, particularly those implemented to control the hazards of the contracted process during operations.

- Require that all subcontractors abide by the same rules to which the contractor is bound by this section.

- Abide by the facility smoking rules.  Smoking is prohibited in certain areas of some facilities.

- Contractors shall attend an orientation program on safety and environmental concerns.  This program must be repeated every 2 years to remain as an authorized contractor.

- DURING CONTRACT WORK, THE CONTRACTOR SHALL:

- Have a designated site safety coordinator present and attentive to the work being carried out at all times that the contractors and/or subcontractors are working at the facility site.

- Ensure that all subcontractors are abiding by the terms of this plan.

- Perform their work while the plant is operating, if necessary, and establish necessary safe practices to permit work under operating conditions without endangering Ward/HMAC's employees and property.  This includes but is not limited to barricading, sign posting, and fire watches.

- Make sure that any equipment, chemicals, or procedures used by the contractor to perform contracted work meet all OSHA, DEP, Ward/HMAC environmental management system requirements.

- Be held responsible and accountable for any losses or damages suffered by Ward/HMAC and/or its employees as a result of contractor negligence.

- Provide their employees with medical care and first aid treatment.  Plant first aid facilities may be used only in case of medical emergencies.

- Use only the plant or building entrance designated, and follow the facility access control practice. The contractor also will ensure that each contractor employee is issued and wears the Ward/HMAC Contractor’s Badge.

- Provide supervisors and employees who are properly trained, including training in all health and safety aspects of the work involved in the contract.

- Provide all personal protective equipment (PPE), and ensure the equipment is in proper working order and employees are instructed in its proper use.

- Maintain good housekeeping in the workplace.

- Follow specific instructions supplied by Ward/HMAC should emergency systems be activated.

- Notify the Health and Safety Department immediately of any injury or illness to contractor employees or subcontractor employees occurring while on the site.  Provide a copy of each accident report to the Health and Safety Department.

- Receive and use a copy of the facility's written safety policies and procedures.

- After conclusion of the contract work, the contractor is responsible for cleaning all work areas and disposing of any discarded materials in a proper, legal and environmentally friendly manner.

# HEALTH, SAFETY AND ENVIRONMENT (HSE)

The Ward Manufacturing Environmental Management System (EMS) is designed to meet the requirements of ISO 14001 Standard.  The principle elements of the EMS and environmental policy are:

- To establish and operate effective procedures aimed at controlling environmental performance to comply with all relevant environmental legislation and regulations;

- To set objectives and targets aimed at achieving continual improvement in environmental reference

- To introduce improvements which contribute to the prevention of the pollution at the source, where possible.

An important part of the EMS relates to the control of contractors and their sub-contractors, who are required to comply with the Ward Manufacturing environmental policies and procedures.

The nature of the contractor activities is such that contractor personnel have significant potential to affect the environmental performance and regulatory compliance of the facility. Contractor personnel and the facility must therefore work together to achieve the facility's environmental policy, the environmental objectives and targets, and the protection of the environment.

Contractors must be aware of the importance of compliance with relevant environmental legislation and regulations, and the consequences of non-compliance.

The contractor is responsible for communicating to all contractor personnel this information.

Ward Manufacturing Environmental Management System

All contractors working at Ward Manufacturing are required to comply with the requirements of the EMS and the environmental policy.   This Environmental Overview provides general details of the Environmental Management System and Environmental Policy.

Environmental Management Basics - Contractors on-site

Contractors shall not allow discharges to drains and/or sewers without prior approval from the Maintenance Manager.

Contractors shall provide adequate spill/release prevention for all bulk materials.

Contractors shall immediately notify the Environmental Department of any spills, releases, or other environmental incidents.

Contractors shall immediately notify the Environmental Department of any abnormal conditions found during excavation at the facility.  Visibly discolored soils, soils with a discernible odor, and/or heavily stained concrete must not be removed from the site without prior approval of the Environmental Consultant.

Contractors shall properly label, store, and dispose of all waste materials generated from their activities.

If Ward Manufacturing personnel are required to work with potentially hazardous materials brought on-site by a contractor, prior approval of the material by the Environmental Consultant and Health and Safety Manager is required.

Contractors must be sensitive to the effects of noise, odor, light, fugitive dust emissions, and traffic movement to the facility and in the local community.

Contractors shall be required to prepare and maintain records pertaining to the work performed in accordance with environmental regulatory requirements, including record retention requirements as necessary.

Contractors shall ensure protection of the natural environment surrounding the work area.

Contractor shall ensure that all employees are properly trained on such things as the proper handling of material and equipment, proper response to incidents involving their material and general information relating to the Ward Manufacturing Environmental Management System.

#### UNSAFE ACTS OR CONDITIONS

If unsafe acts or conditions are observed, Ward/HMAC will require that corrections are made immediately and may stop work.  Consequences of any such work stoppage shall be borne by the contractor or subcontractor with no recourse against Ward/HMAC or Ward/HMAC employees.

#### HSE TRAINING AND MEDICAL EVALUATION

Each contractor is responsible to ensure its employees and its subcontractors have received all HSE training and medical evaluations required by federal, state and local regulations.

Copies of training and certification records must be maintained by the contractor and subcontractor and will be made available to Ward/HMAC upon request.

#### HSE PERFORMANCE REQUIREMENTS

The HSE program and performance demonstrated by contractors and subcontractors is part of the selection criteria in Ward/HMAC awarding of initial and future contracts.

- ACCIDENT PREVENTION PROGRAMS

- JOB SITE INSPECTIONS

You, as the contractor, are responsible for monitoring safety on your project(s).  Deficiencies, such as, unsafe behavior or unsafe conditions shall be corrected immediately.  The Health and Safety Department of Ward/HMAC will make periodic inspections to assure that all safety procedures and practices are being adhered to.

- ACCIDENT AND INCIDENT INVESTIGATIONS

You, as the contractor, are responsible for conducting a thorough investigation for both accidents and incidents.  Accident and incident forms must be properly filled out and forwarded to the Ward/HMAC Health and Safety Department.  All accidents, no matter how minor, must be reported to the Health and Safety Department of Ward/HMAC.

- SAFETY MEETINGS

Safety meetings must be held at least once every week with written documentation of the topic and attendance.  A copy of the meeting topic along with attendance documentation must be forwarded to the Health and Safety Department of Ward/HMAC.

- MEDICAL / FIRST AID PROCEDURE

Contractors providing services to Ward/HMAC are responsible for medical and first aid for their employees.  Provisions for medical treatment of injured workers must be made prior to beginning of the project along with documentation of the contractor’s Panel of Practitioners to the Health and Safety Department of Ward/HMAC.  Accidents must be reported to the Health and Safety Department of Ward/HMAC by use of the Contractors Injury and Illness Report supplied by the Health and Safety Department of Ward/HMAC.  Contractors are responsible for bills incurred for medical treatment and recording of injuries and illnesses on their OSHA log.

The contractor may arrange to use the Health and Safety Department of Ward/HMAC’s medical facilities prior to beginning of project.  The Health and Safety Department of Ward/HMAC will provide medical treatment within their parameters.  If medical treatment is required above and beyond what can be given by Ward/HMAC’s Occupational Health Nurses, they will refer to the contractor’s Panel of Practitioners.

- HOUSEKEEPING

Ward/HMAC requires housekeeping to be part of each individual’s job.  The following rules are enforced at all facilities.

- Keep work areas neat and orderly.

- Keep all emergency exits clear and unobstructed.

- PERSONAL PROTECTIVE EQUIPMENT

Personal protective equipment shall be provided by the contractor for their employees.

- EYE/FACE PROTECTION

- Eye Protection

- ANSI approved safety glasses with side shields are required in all facilities of production.  Eyeglasses not required in offices, lunchrooms or restrooms.  Additional eye protection is required to perform such processes as burning, grinding, etc.

- Employees who are required to wear prescription glasses must have glasses fitted with side shields.

- Dark safety glasses are not permitted in our facilities.

- Burning goggles

- Burning goggles or shaded safety glasses with a shade 5 lens are required for all burning and cutting operations.

- Face Shields

- Are required when grinding, handling chemicals such as acids or caustics, and molten material.

- Safety glasses must be worn under face shields.

- Welding hoods

- Welding hoods shall be kept in good condition and proper shields in place.

- Safety glasses must be worn under hoods.

- HEARING PROTECTION

- Approved hearing protection is required in most all facilities.  The supervisor, engineer in charge of the project, or the Health and Safety Department of Ward/HMAC will inform you of areas where hearing protection may not be required.  Hearing protection is not required in offices, lunchrooms or restrooms.

- Hearing protection is required when operating such equipment as jackhammers, etc.

- HAND PROTECTION

- Gloves shall be worn when handling rough, sharp-edged material.  Gloves shall not be worn around machinery where gloves are likely to become entangled.

- Special gloves are required in processes such as welding, working with chemicals or corrosives.

- FALL PROTECTION

- Fall protection is required to be worn when working at heights greater than four feet unless protected by standard handrails.

- Safety belts are not acceptable as fall protection.

- Safety harnesses must be visually inspected prior to each use.  Harnesses must be removed from service if subject to fall, heat exposure that causes damage to webbing, or defects such as tears, cuts are identified.

- Safety harnesses are required in the operation of aerial lift platforms.

- FOOT PROTECTION

ANSI approved steel-toed shoes/boots are required to be worn at all times.

- CLOTHING

- Shirts are required at all times.  Shirts must have a minimum sleeve length of 4 inches.

- Trousers must extend below ankles.

- Rings and jewelry shall not be worn when operating power tools or near rotating equipment.

- When handing acids or corrosives, chemical aprons or apparel such as suits must be worn.

- HEAD PROTECTION

- Hard hats with American National Standards Institute (ANSI) approval are required at all times in most facilities.  Check with the Health and Safety Department of Ward/HMAC eaHeatH or the contact person for your project.

- Metal hard hats are not permitted in the facilities.

- Do not alter suspension of hats.

- TOOLS

All tools, both powered and hand tools, must be kept in good repair.

- HAND TOOLS

- Inspect all tools prior to using.

- Keep hand tools in good condition.

- Do not leave hand tools on walkways, scaffolds, ladders or overhead working levels.

- Do not throw tools from one level to another or from one employee to another.

- PORTABLE POWER TOOLS

- Portable power tools must not be operated by untrained individuals.

- Electrical cords must be double insulated or shall be of an approved system that contains three wires with a ground.

- OSHA 29 CFR 1926.404 requires inspection and testing of equipment.

- The use of ground fault circuit interrupters is required.

- All guards and shields must be in place and in good condition.

- Electrical power tools are not to be used in explosive atmospheres unless tools are approved for service in a hazardous atmosphere.

- All defective tools must be taken out of service immediately.

- EXTENSION CORDS

- Extension cords must be of the three-wire type and protected by a ground fault interrupter or plugged into a ground fault interrupter receptacle.

- Extension cords must be protected from sharp edges and traffic.

- Extension cords and connections must be kept away from water and damp surfaces.

- OSHA 29 CFR 1926.404 requires inspection and testing of equipment.

- EQUIPMENT

- GENERAL

Each employee is responsible to inspect equipment before use.  Defective equipment must be removed from service immediately.

- LADDERS AND SCAFFOLDS

- General

- Metal or aluminum ladders are not permitted.

- Defective ladders must be removed from service immediately.

- Straight / extension ladders

- Ladders are to be erected using the 4:1 ratio (for every 4 feet of the ladder height, the base will be placed 1 foot from the vertical.

- Ladders must be secured at both the top and the bottom.

- Ladders must be equipped with safety feet.

- Ladders shall never be used in a horizontal position as scaffold or a platform.

- The length of single ladders or individual sections of ladders shall not be greater than 30 feet in length.

- Two section ladders shall not exceed 48 feet in length and over two sections ladders shall not exceed 60 feet in length.

- Extension ladders shall be equipped with positive stops.

- Ladders used to gain access to a roof or other area shall extend at least 3 feet above the point of support.

- Ladders must be a Type 1A, 300 pound duty rating or Type 1AA 375 pounds.

- When ascending or descending, the climber must face the ladder.

- Stepladders

- Do not use stepladders in a folding position.

- Do not stand on the top 2 steps of a stepladder.

- Stepladders shall not exceed 20 feet in length.

- Ladders must be a Type 1A, 300 pound duty rating or Type 1AA 375 pounds.

- Scaffolds

- All scaffolds used by contractors and subcontractors must be designed by a qualified person and erected by trained workers under the direction of a competent person.

- A competent person must inspect scaffolds before each work shift.

- Erect scaffolding on sound, rigid footing.

- Scaffolding must be equipped with handrails, midrails, toe boards, screen between the toe board and the guardrail, extending along the entire opening consisting of number 18 gauge U.S. Standard Wire one-half inch mesh or equivalent, where persons are required to work or pass under scaffolds and deck boards.

- Scaffold planks without hooks must extend a minimum of 6 inches over the end supports.  All scaffold boards are to be cleated on each end and be scaffold grade lumber or metal scaffold boards.  Manufactured planking with hooks may be used and attached to the scaffold structure.

- Full planking the full width is required at the working level.

- Provide an access ladder for all scaffolds.  Climbing the side of scaffolding is not permitted.

- The entire scaffold must be tied off and securely braced at height intervals not to exceed 30 feet horizontally and 26 feet vertically.

- When using suspended scaffolding, attach and secure a safety harness before stepping on the platform, and do not remove it until clear of the scaffold.  Tie off to an independent lifeline or building structure.  Use one lifeline per person.

- SIGNS AND BARRICADES

- At locations where potential hazards exist, contractor personnel shall be responsible for posting, installing and maintaining signs and barricades to detour or warn person.

- Barricades must be 42 inches high.  Barricades shall be kept 6 feet from the edge of excavation, holes, platforms and roofs.

- Contractor employees shall obey all signs, signals and barricades that are posted to warn of potential or existing hazards.

- RIGGING

- Inspect hooks, shackles, beam clamps and slings before each use and remove defective items from service.

- Have safety latches on all hooks.

- Do not leave suspended loads unsecured or unattended.

- Inspect synthetic slings for wear indicator threads, heat damage and eyelets torn prior to use.

- Destroy damaged sling immediately.

- COMPRESSED AIR

- Compressed air used for cleaning purposes must not exceed 30 psi.  Wear goggles or face shields over safety glasses when conducting cleaning.

- Compressed air is not to be used for blowing material off yourself or others.

- Keep hoses off the ground or floor wherever they interfere with walkways, roads, etc.

- MOTOR VEHICLE AND POWER-OPERATED EQUIPMENT

- Trucks and automobiles

- Facility speed limits and other regulatory signs must be obeyed.

- Pedestrians always have the right of way.

- Passengers in the rear of pickup trucks must be seated within the confines of the bed of the truck.

- Riding on the side or tailgate of a pickup truck in prohibited.

- All vehicles used during a project for contract activities must be equipped with an operable backup alarm.

- Cranes

- Cranes are to be operated within the design limits of the manufacturer.

- The operator of the crane must hold a current certification to operate said equipment.

- The rated load shall never be exceeded.

- Cranes shall be properly labeled with load capacities, warning signs and instructions.

- All accessible areas within the radius of the counterweight swing must be barricaded to limit access.

- Personnel are prohibited from riding on the hook or “headache” ball.

- Crane inspections must be conducted on equipment per the OSHA standards.

- When personnel baskets are used, all OSHA requirements must be followed.

- Lift Trucks

- Only authorized and trained personnel are permitted to operate lift trucks.

- Riders are not permitted.

- Lift trucks must be equipped with horns, backup alarms, headlamps and amber rotating or flashing lights.

- FIRE PROTECTION

- ALARM / NOTIFICATION

- The Health and Safety Department of Ward/HMAC will instruct you on alarm notification at the Contractor Safety briefing prior to the project beginning.

- Report any fire through floor supervision.

- Familiarize personnel with emergency exits and points to assemble for a head count.

- FLAMMABLE / COMBUSTIBLE LIQUIDS

- Flammable liquids must be stored in a building, cabinet or area remote from general access or traffic.  The storage area must be properly marked.  No smoking, matches or open flames or other ignition sources are permitted within 50 feet of the storage area.

- Flammable liquids stored inside a building must be stored in an approved flammable storage cabinet, properly grounded and supplied by the contractor.

- Flammable liquids must be stored and covered in their original container or safety cans and properly identified.

- Drums used for dispensing must be properly grounded.  Self-closing valves must be used for dispensing.

- A type ABC fire extinguisher, supplied by the contractor, must be readily available.

- All rags, trash and other material contaminated with flammable liquids are to be disposed of daily in a separate metal container with a lid.

- JOB SITE PROCEDURES

- BURNING AND WELDING PROCEDURES

- All electric welding must have proper grounding.

- Do not leave a rod in the electrode holder when you lay it down.  Place stub ends in proper container, not on the floor.

- All arcs should be shielded in production areas.

- Approved welding helmet must be worn.

- Before connecting regulators to cylinders, carefully open cylinder valve a crack to blow out foreign particles.  After regulators are connected, stand to one side of the gauges while the cylinder valve is opened.

- Cylinders must be secured in vertical position with valve caps in place.

- Keep oil and grease away from oxygen regulators, hoses and fittings.  Do not use oxygen to blow clothing clean.

- Never use oxygen to operate pneumatic tools.

- A type ABC fire extinguisher, supplied by the contractor, must be readily available.

- HOT WORK PERMIT

- Before welding in an area where there are combustibles or flammables, a hot work permit must be obtained from the Maintenance Department of Ward/HMAC.

- Hot work permits must be posted in the area where the work is being performed.

- The hot work permit is only good for 1 shift.

- A fire watch must be positioned during the welding or burning operation and 30 minutes after.

- A type ABC fire extinguisher, supplied by the contractor, must be present during procedure.

- CONFINED SPACE PERMIT

- A confined spaces permit must be obtained from the Maintenance Department of Ward/HMAC prior to entry.

- The following precautions must be taken:

- The confined space must be empty.

- All equipment must be in a state of ZERO energy.  Equipment must be locked and tagged out.

- The confined space must be ventilated.

- Atmospheric testing must be performed.

- An attendant must be positioned at the entrance of the permitted space at all times.  This individual must maintain communication with entrants at all times.  The attendant must not attempt to rescue until help arrives.

- The permit must be posted at the confined space entrance.

- Permits are only valid for one shift.

- Ground fault circuit interrupters must be used at all times.

- It is the responsibility of the contractor to obtain the confined space permit.

- ELECTRICAL WORK

The contractor is responsible to ensure its employees and subcontractors comply with all applicable electrical safety requirements.

- Each contractor must comply with: 29 CFR 1910 Subpart S, 29 CFR 1926 Subpart K, National Electric Code (NFPA 70), Electrical Safety Requirements for Employee Workplaces (NFPA 70E), local codes and requirements, Ward/HMAC engineering standards and practices.  The contractor and subcontractor shall be responsible for providing “Qualified Persons” as defined by the above regulations.  Electrical contractors working for Ward/HMAC must include, in their HSE program document, a description of how they train and utilize “Qualified Persons”.

2.	Qualified Person/Electrical Technicians

Qualified persons and electrical technicians are those employees who are permitted to work on or near exposed energized parts.  These individuals shall, at a minimum, be trained in and be familiar with the following:

- Shall be capable of working safely on energized circuits and shall be familiar with the proper use of special precautionary techniques, personal protective equipment, insulating and shielding materials and insulated tools.

- The skills and techniques necessary to distinguish exposed live parts from other parts of electric equipment.

- The skills and techniques necessary to determine the nominal voltage of exposed live parts.

3.	Electricians / Technicians are required to wear the following PPE as a minimum,

Reference NFPA 70E:

- Safety glasses with side shields.

- Hearing protection.

- Steel toed footwear.

- Flame retardant shirt and pants rated at 8 cal/cm2 (An untreated cotton T-shirt must be worn under Flame Retardant long sleeve shirt).  NOTE: Sleeves may be rolled up when performing non-electrical tasks.  Contractor/Vendors may utilize a coverall rated at 8 cal/cm2.

- Head protection with a Class E rating.

- Additional PPE may be required when the employee is assigned a task.  The employee or supervisor is responsible for reviewing the Hazard Risk Category Classifications and Protective Clothing and Personal Protective Equipment Matrix from the NFPA 70E Standard to determine appropriate requirements.

All Hazard/ Risk Category 3 and 4 tasks require Electrical Switching Clothing.

Two employees are required to be suited at this time.

- LOCKOUT / TAGOUT PROCEDURE

Lockout / tagout procedures are implemented to prevent unexpected release of energy.

- All energy sources with electrical, chemical, thermal, pneumatic, hydraulic and mechanical potential must be locked and tagged out prior to work activities.

- Locks must be placed on equipment by the contractor’s employee and the Maintenance Department of Ward/HMAC notified of such.

- Energy controls must be verified prior to working on equipment.

- Removal of locks must be completed as soon as work is completed.  The Maintenance Supervisor of Ward/HMAC shall be notified at the time and witness the removal.  Equipment shall not be energized until verification is made that no personnel are in danger of the startup of equipment.

- RESPIRATOR PROTECTION

Respirators shall be worn when working with or exposed to gases, vapors or dust is above the OSHA permissible levels or when required by Ward/HMAC.

- Employees must be trained in care and usage of respirators.  It is the responsibility of the contractor to train their employees.

- Respirator users must be fit tested according to 29 CFR 1910.134.

- HAZARDOUS COMMUNICATION PROGRAM

- Contractor will be informed by the Health and Safety Department of Ward/HMAC of the availability of material safety data sheets and their locations.

- Contractor must inform the Health and Safety Department of Ward/HMAC of any hazardous conditions resulting from operations conducted at the facility.

- Contractor is required to submit all material safety data sheets for all materials brought into facility and inform the Health and Safety Department of Ward/HMAC of any hazards associated with the chemicals and their use.  All containers must be properly labeled when used on worksite.

- Contractor employees must be knowledgeable of appropriate work practices, emergency procedures and personal protective equipment needed when working with hazardous chemicals.

- EXCAVATIONS

- Excavations and trenches must be barricaded to protect pedestrians and vehicles.  All excavated material must be piled at least 3 feet from the edge of the excavation.

- Depths of 5 feet or greater must be shored or sloped to eliminate the hazard of cave-in.

- A ladder for employee exit and access must be provided.  An employee working in a trench must not have to walk more than 25 feet to gain access to the ladder.

- The atmosphere in the trench is to be tested prior to entrance when the possible presence of harmful gases or a state of oxygen deficiency exists.  Continuous monitoring may be necessary.

- STEEL ERECTION

- The area below steel erection activities must be barricaded to prevent access by unauthorized personnel.

- A hoisted steel member shall not be released until it is anchored by at least two bolts or sufficient weld is placed on steel member.

- Tag lines must be used to control loads.

- Personnel are prohibited from climbing columns, walking on beams, traversing the trusses and sitting on top of columns unless fall protection is worn.

- Tools and containers for rivets, bolts or welding rods must be secured to prevent falling.

- DEMOLITION

- Prior to permitting employees to start demolition operations, a survey shall be made of the structure to determine its condition.

- The area must be barricaded to prevent unauthorized personnel from entering.

- Any asbestos containing material must be removed according to local, state and federal regulations.

- INSULATION

- Any unknown material is to be treated as asbestos containing material.

- Contractor personnel are prohibited from removing or disturbing material until the insulation is determined to be asbestos free.

- Asbestos containing material must be removed in accordance with state and federal regulations.

- GENERAL SAFETY RULES

- Horseplay of any kind is absolutely forbidden in the facility.

- Do not walk or stand under or beside suspended loads.

- When discarding boards, always remove nails or bend them down.

- Welding curtains must be used around welders to prevent other employees from receiving arc burns.

- The use, possession, sale, transfer or purchase of alcohol, illegal drugs or controlled substances on Ward/HMAC property is prohibited.

- Firearms and other weapons are forbidden in the facilities.

- Exposed vertical reinforcing rods must be provided with protection such as plastic caps, 2 x 4 lumber, etc. to prevent impalement of personnel.

- Contractor employees will be expected to adhere to all other established safety rules / policies developed by Ward/HMAC.