Machine Connection Notes for Paulo Products

Note for all Reducer Hoods:  Reuse the existing legs. If can not be reused, need flatbar from Home Depot and bend in “L’ shape.

Note:   Two 8” 30 deg elbows were shipped per drop in case an offset is needed to go out through the roof.

934 / 933:   Already is connected with clamp together duct.  Will run the two 8” pick ups into one 12” duct and run 12” duct through the roof.

Note:  reuse exiting 12”/8”/8” branch, 12” pipe (4pcs), 12” 90 (1pc) and 12” 45 (1pc).  New 12” clamps and nipples were ordered, but reuse existing if possible.

931:   Currently goes straight out.

932:   Currently goes straight out.

927:   Offsets towards back of oven about 10”.  Will need two 8” dia  30 deg elbows.

928:   Offsets towards back of oven about 10”.  Will need two 8” dia  30 deg elbows.

Currently has 12” dia duct.  We will run 8” dia. Duct.

930:    Currently goes straight out.

926:    Currently goes straight out.

5226:   Currently goes straight out.

1030 and 1032:  All ductwork will run inside the building.  The two 8” diameter drops

will duct together to one 10” diameter drop.  Keep ductwork high as

possible and to the back wall.