Greenville Tech Punch List

1.  Install new spring and hood on Oskar arm

2.  Mastec ALL bolted joints on the outside.

This includes where the flanges bolt to fan, dust collector,

silencers, diverter valves.  Also, where the 24” duct is bolted to the

curb cap.

Also, Mastec the	 inside of each dust collector at the hopper / filter section joint.

3. When done spray paint the rusted parts on the collectors and fans Oskar Blue

4.  Flash the ductwork penetrations with metal flashing. Make sure to caulk to keep from leaking.

Outside:

Two 18”dia duct from the grinding tables

One 18”/18”/14” ductwork from the booths (Pan type)

One  40”x 32” square duct / return air for booths

Inside:

One 18”/18”/14” duct from booths (flat type)

Two  18”dia duct from grinding tables

5.  Install Offset to the inlet plenum for the 14” duct (booths)

Will have to connect 14” angle flange adaptor and then duct into the building. Install 14” 90 deg elbow and then 14” endcap

6. Install 30” diameter 30 degree elbow on the diverter valve branch part

7. Install 24”diameter 30 degree elbow on the diverter valve branch part

8. On the grinding duct on the roof, seal the 24” clamps that hold the 24” diameter 90 degree elbow. Also, pin any clamps that do not have a pin on the clamp.

9.  Give the two Electrical Control Boxes to Stewart