Scope of Work for Unytite

Unytite

One Unytite Drive

Peru, IL  61354

Ron Pioli

P# 815-224-2221 ext 210

Installation of Three ODR 3000 and ductwork and hoods

First Trip (on 12/17/09 – 12/18/09)

- Set three ODR 3000 side by side and connect together

- Attach the 20” plenum box to the right hand unit

- Give three control panels to Ron Pioli, they will mount

See attached drawing for ductwork layout and ODR 3000 connection details.

They are closed on the weekends, 12/24/ 12/25/ 1/1 and 1/2.

Second Trip (12/28/09  - 12/30/09)

- Install ductwork and hoods per the drawing

You will need to provide your own hardware (nuts, bolts, etc.)

You will need to provide the unistrut, beam clamps, hangers.

I will send the silicone with the Absolents.

You will need to apply a bead of silicone in each clamp

Make sure the clamp handle is at the top of the ductwork so it can  not be seen from the floor.