山貿科技有限公司			SAN MAO TECHNOLOGY CO., LTD

台中市神岡區大明路191巷92號 No.92, Lane 191, Daming Rd., Shengang District, Taichung City 429, Taiwan

Tel: +886-4-2526-9689 	Fax: +886-4-2526-9860

E-mail: sales@smartvent-technology.com  Website: http://www.smartvent-technology.com

September 02, 2011

## ORDER CONFIRMATION

INVOICE NO.  P-1000902-002

PO NO.       3534

INVOICE of	 Large Dust Collectors, Blowers and Downdraft Tables

For account and risk of Messrs.  AVANI ENVIRONMENTAL INTL. INC.

95 Cypress Drive, Youngsville, NC 27596

Shipped by: 	SAN MAO TECHNOLOGY CO., LTD.

From:	Taiwan      to  USA

SDC large cartridge collectors and blowers

FOB TAIWAN

| MSI / ACR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Model | Description | Unit Price | Q’ty | Total |
| 1 | SDC-15-16 | Include: - SDC-16 cartridge collector x 1 pc - DBN-150 blower on the machine top x 1 pc - DBN-150C control box x 1 pc |  |  |  |
|  | SDC-16 | Cartridge Collector (dust filtration only), Drum Type - Inlet: Dia. 16” (angle flange, will provide inlet square to round) - Cartridge Filter: Ø325mm x 1320mm x 16pcs (Dia. 12.8" x 52" x 16pcs) - Material: Polyester with TEFLON coating - Filter Area: 19m2 x 16 = 304m2 (3271.04 ft2) - Pleat Depth: 48mm - Horizontal Positioned Filters - UL certified control panel, wiring conduits, and solenoids. - Continuous Welded Seams - Gasket and Latched Doors - Powder Coated Paint and Reverse Jet Pulse Cleaning - Include: - Drum & Jack Lifting Tool - Filter Chamber - Hopper - Inlet Square Pipe (need dimensions) - Fan/motor combinations - No. of Valves: 4 - Inlet Air: 3/4" - Compressed Air Requirement: 90 PSI - Compressed Air Reservoir: 15.8 gallon - Dimension: 88.4" x 96.1" x 216.5" (2244 x 2440.1 x 5495mm) - Construction: 13 Gauge for panel; 11 Gauge Steel Cabinet - Max. Operating Temp.: 120℃ - Paint Color: Blue - Logo: Avani **Hole for explosion vent: 24”W x 36”H x 2pcs on the right. **Pulse control box & blower control box are on the left. **Reference: SDC-15-16 submittal (explosion vent) 2011.09.01 | 11,760 | 1 | 11,760 |
|  | DBN-150 | 15 HP / 4 Pole (on the machine top) 230/460V / 60Hz / 3Ph (connect to: 460V) Connecting Dust Collector CFM @ Dust Collector Inlet. @ 8” sp. 6400. | 2,850 | 1 | 2,850 |
|  | DBN-150-C | UL Control Panel for 15HP blower & SDC-16 Pulse Control Box: run off of 120V, 1- phase, No transformer | 610 | 1 | 610 |

SUBTOTAL: US$ 15,220.-

Downdraft Tables

FOB TAIWAN

| MSI / ACR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Model | Description | Unit Price | Q’ty | Total |
| 2 | SDT-180 | Custom Downdraft Table - Outlet: Dia. 6.8” OD - Rubber Table Top - Gap Between Table Joints - One Set Includes: - left table x 1 pc - middle table x 1pc - right table x 1pc - Paint Color: Blue - Logo: Avani **Reference: MSI - SDT-180 2011.09.01 | 2,500 | 2 sets | 5,000 |

SUBTOTAL: US$ 5,000.-

TOTAL AMOUNT for MSI: US$ 20,220.-

Remark:

- PAYMENT: 35% down payment remitted before shipment; another 35% wire transferred when the container is approaching to US; 30% balance cleared one month after ETA date of the container.

- SHIPMENT: 45 days after confirmation of O.C. With any available container closer to the delivery time to NC.

- VALIDITY: 6 months.

- PACKING: Our standard packing.

Bank:

MEGA INTERNATIONAL COMMERCIAL BANK CO., LTD FENG YUAN BRANCH

519 CHUNG CHENG ROAD FENG YUAN, TAIWAN, R.O.C

TEL: 886-4-25285566       FAX: 886-4-25206431

SWIFT ADDRESS: ICBCTWTP 035

Account NAME: SAN MAO TECHNOLOGY CO., LTD

Account NO. 035-585-4620

SAN MAO TECHNOLOGY CO., LTD    AVANI ENVIRONMENTAL INTL. INC.

CHARLES CHENG        				CHARLES SITHES

General Manager							Product Manager