Agfa Inc.

Avani  Environmental Int’l  Mechanical Labor and Handling

- Uncrate ODF-1000 from Sweden

- Cut and Install leg extensions (10”) over existing stub legs.  Drill through to allow for a ¼ - 20 bolt to be placed and a lock-nut to be placed inside.

- Remove 2 filter cassettes and verify filter one (S10B3/ 595) filter 2 TRSA-N 990.  Remove Absolent part numbers and identifications (remove plates and rivets) Replace with decals or new plates provided from Agfa.

- Remove owner’s manual and replace with Agfa manual.

- Remove serial number plate and replace with Agfa plate.

- Install Agfa labels on top door as shown on drawing.

- Remove junction box.

- Install ventilation collar over the hole on the LEFT side of the unit using self tapping screws.  Seal any possible holes.

- Install steel elbow and 45 degree valve for drainage with a downward slant for better drainage.

- Install new electrical box and make all necessary connections.  Ensure that the 15’14/5 wire is attached to the electrical box via proper stain relief  for on site connection.

- Install the charcoal filter vent hood onto the unit using a thin bead of clear silicone on top outside  of provided screw holes making sure we have full coverage around the full perimeter.  Use 16M 5 x 14 screws and nuts using large washers (5/8”) and use a small amount of Blue 425 medium strength Loctite on each screw.  Place a small bead of silicone on the inside upper portion of the vent hood. Clean off excess caulking creating a smooth edge on outside edge.

- Remove door from charcoal vent hood and install gasket as shown on drawing.

- Install orange sticker on charcoal filter access door.

- Static pressure gauge install brass barb fitting in top location only

- Duct assembly Install static pressure gauge bracket and gauge.

- Duct assembly install brass barb fitting for static pressure tube to inside radius of 90 degree elbow

- Duct assembly install plastic tubing from brass barb on static pressure gauge to brass barb fitting on elbow.

- Connect unit to electrical power.

- Check fan rotation and record.

- Check for vibration and record.

- Check amps and record.

- Hypot test unit and record.

- Check control panel on/off, lights, disconnect and overall appearance and record.

- Check the both Absolent gauges and record  Static Pressure.

- Check Static Pressure on I duct adjust fresh air damper to correct reading and record.

- Test and record Airflow and record.

- Check for air leaks and record.

- Adjust door latches and tighten nuts on latch.

- Install charcoal filter making sure it is in a plastic bag and record

- Check fit and finish of complete unit and record.

- Bubble-wrap the collector while standing.

- Lay collector into new crate. Insert I duct assembly.  Raise sidewalls and assemble.  Band collector crate to insure stability for freight carriers.

- Provide complete check-list upon completion.

____________________________________________________________________________________________