May 2 2012

Infasco

700 rue Ouellette

Marieville Quebec

J3M 1P6

Quote number:190412 REV 1

Add qty 1 ODR3000 plus additional ductwork for BV6-03

Reference: Mist collection for BV6-03 & NB520-01

We appreciate this opportunity and are pleased to offer a solution for the capture of mist from your Cold Headers.

Absolent is successful at collecting oil from cold headers, with no maintenance for up to six years.

Below is the description of the units that will be used at your facility.

# Design Criteria:

# The following criteria are the base for the recommended equipment.

# Required Airflow:

# Coolant Liquid: Straight oil

- Contamination: Sticky black smoke

# Inlet Load: 50-120mg/cu.m.

- Number of pick-up points

# Type of Machine:

## BV6-03

Below is the quotation for qty of 3 ODR3000 for BV6-03.  This system will properly collect the oil mist and smoke. Improve house keeping and workers safety and comfort.

This header will require qty of 3 ODR3000 joined together delivery at total CFM of 7,050cfm.

Ductwork is designed to maintain a transfer velocity of 3000fpm with ductwork sloped towards the pick-up points.

Pick-up points will be;	qty of 2 8” dia horizontal into header cavity 									qty of 1 8” dia. in main cavity of enclosure

qty of 1 8” dia above control panel

qty of 1 8” dia near exit door at finished product conveyor

qty of 1 6” dia with a hood and curtains over the waste bin							qty of 1 6”dia over product exit conveyor and hood 								qty of 1 6” dia. Over hood for cooling area of parts.

qty of 1 4” dia on finished parts conveyor

qty of 1 4” dia on waste conveyor

Clamp together Ducting System

- QF Pipe, Adjustable Nipples, Elbows, Branches at 45 degree, Reducers

- QF Flange Connections, Header Adapters, Clamps

- Quick disconnect for all flex hose connections

## PRICING Qty 3 ODR3000, Ductwork and Qty 2 Hoods

Price FOB manufacturer, Taxes Extra.................................................................$81,356.00

PRICING ADDER for ODR3000 located in corner.

Price FOB manufacturer, Taxes Extra.................................................................$2,600.00

## PRICING ESTIMATE INSTALLATION

## This price is an estimate using local contractor....................................................$3,000.00

## NB520-01

This header will require qty of 2 ODR3000 joined together delivery at total CFM of 4,700cfm.

Ductwork is designed to maintain a transfer velocity of 3000fpm with ductwork sloped towards the pick-up points.

Pick-up points will be;	8” dia over header cavity 												8” dia. over control panel 												6” dia with a hood and curtains over the waste bin								6”dia over product exit conveyor and hood 									5” dia. Over hood for cooling area of parts.

Clamp together Ducting System

- QF Pipe, Adjustable Nipples, Elbows, Branches at 45 degree, Reducers

- QF Flange Connections, Header Adapters, Clamps

- Quick disconnect for all flex hose connections

## PRICING Qty 2 ODR3000, Ductwork and 1 Hood

Price FOB manufacturer, Taxes Extra.................................................................$52,907.00

## PRICING ESTIMATE INSTALLATION

## This price is an estimate using local contractor....................................................$2,400.00

## Recommended Equipment

# Absolent ODR-3000-Basic,  Mist & Smoke Collector

Configuration:	Vertical

Motor: 	(1) 10.0 HP (Direct Drive Fans)

Dimensions: 	L 44.9 " x W 46.9" x H 140.8"

Inlet: 	12.4” (Bottom plenum left or right)

Outlet:	Top of the unit. Outlet connection accessory

Weight (Clean filters): 2205 Lbs.

Air Volume: 	2350 CFM

Noise Level:  	73 dBA @ 3 Meters from Outlet Grille

Electrical: 	460 or 575 volts , 3-Phase, 60Hz  (Junction Box)

Filtration: 	Fiber Bed 1:	258 Sq. Ft.

Fiber Bed 2:	275 Sq. Ft.

Absolute Filter:	607 Sq. Ft.

Filter access:	Front Load Filter Accessibility

Enclosure	NEMA 12 Enclosure

Gauges:	Magnehelic Gauges/ 0-10": (3 Total)

Control Box: 	Not included

# Absolent ODR-3000 Floor Stand

- Approximately 18” Height

- Custom Painted (Pittsburgh Paint Safety Blue)

Performance Guarantee:

Avani Environmental  along with Absolent will guarantee fiber bed one and two, and the final HEPA filter for a period of one year minimum performance.

Freight/FOB:			ODR-3000’s Abslolent (Norfolk, VA)

Ducting (Thomasville, NC/ Avani Warehouse, Youngsville,        NC)

Materials (Avani Warehouse & Manufacturers)

Terms (US FUNDS ):		1/3 with PO, 1/3 equipment on site, Balance Net 30 Days

Testing:	Avani  will provide testing (start-up and after 90 days) at the inlet and readings after each fiber bed, also a final reading after the HEPA.  All readings are calibrated and can be downloaded in a windows base program.

Drawings:	Complete drawings will be generated within seven days after receiving a purchase order.  All drawings must be approved prior to any fabrication of ducting and/or custom features.

Again, I appreciate the opportunity to present this proposal for your consideration.  Please call me if I can be of further assistance.  Avani Environmental  looks forward to working with you on this project.

Respectfully,

Bill Strouth

Regional Sales & Systems Engineering

AVANI Environmental Int'l, Inc.

Wasaga Beach Ontario                                                                                                                                                                                    Cell 519 546 3340        Head office phone 919-570-2863      Fax 919-570-2863

email: bstrouth@rogers.com

Visit our Website at: www.avanienvironmental.com