Customer:	Johnson Controls

76 Armstrong Road

Battle Creek, MI 49037

Contact:	Doug Piccard

Phone: 	269-832-0178

Email:

Date:  4/11/13

Project #:

Sales Order #:

Serial #: 130602-02, 130603-02

Model #: RM-6000

AEI Crew:  Bill Boisvert

SCOPE OF PROJECT:        Installation – Service – (Warranty) – Demo – Delivery - Engineering

- RM-6000, SN:130602-02 (Cell 8)

  - Remove old paper from collection bar.

  - Ran collection motor with no media.

  - Motor just barely ran.

  - Adjusted Speed control.

  - Installed roll media and ran, with fan in operation, by using Collection/Delivery push button.

  - Check control set points.

    - Ideal time found at 30 minutes – left as found

    - Travel time found at 2 minutes – reset to 7 minutes

  - Placed fan and RM-6000 in operation.

  - Ran through several cycles with no problem.

- RM-6000, SN 130603-02 (Cell 9)

  - Remove old paper from collection bar.

  - Ran collection motor with no media.

  - Motor ran fine.

  - Made slight adjustment speed control.

  - While loading roll media found one spoke on media transport missing hardware on one side.  Repaired using parts from old RM-6000’s

  - Installed roll media and ran, with fan in operation, by using Collection/Delivery push button.

  - Check control set points.

    - Ideal time found at 30 minutes – left as found

    - Travel time found at 2 minutes – reset to 7 minutes

  - Placed fan and RM-6000 in operation.

  - Ran through several cycles with no problem.

- Filter Inspections

  - Cell 9

    - Found one filter had parted from top cap – Replaced with new filter.

    - Found that pulse air pressure had been reduced to 70PSI to keep from damaging filters.

    - Filters are not cleaning as they should.

    - Reset pulse pressure to 90 PSI

    - Filters seemed to be cleaning better.

    - Recommended that collector be pulsed off line as often as possible.

  - Cell 8

    - All filters ok

    - Same problems with cleaning as cell 9

- RM-6000 Running inspections

  - Inspected roll media at the following times.

    - 4:30pm 4-10-13 – OK

    - 9:30pm 4-10-13 – OK

    - 10:30pm 4-10-13 – OK

    - 5:30am 4-11-13 – Both units had media bunched up on both sides of the unit.

    - Removed roll media from both units.

  - Monitored air flow in Cell 9

    - 4:30pm

      - 3237 FPM

      - 2925 FPM

      - 2965 FPM

    - 9:30pm

      - 2936 FPM

      - 2925 FPM

      - 3237 FPM

    - 10:30pm

      - 2940 FPM

      - 3003 FPM

      - 3199 FPM

    - 5:15am

      - 2965 FPM

      - 3199 FPM

      - 3062 FPM

Note: Air Flow position 1 was lower right intake (from inside cell), position 2 was over door, and position 3 was lower left intake.

The roll media appears to be being pulled in under the chain drive.  Once this occurs the media tears and clogs the system.

Thanks,

Bill Boisvert

Bill Boisvert - Cell (919) 971-3910

Project Manager – Avani Environmental

E-mail bboisvert@avanienvironmental.com

CUST0MER REMARKS: _____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Customer Signature: _________________________________________Date:___________________

Title or Department: ________________________________________________________________