Sun Hydraulics / Sarasota, FL                            5/30/08

Absolent ODF 2000 Unit #____

1.  Hanging height above hood:  16”   yes  or  no

If not, what is the height ___

2.  Does Absolent hang vertical:  yes  or  no

If not, straighten.

3.  When looking at he ductwork system from the end

view, is it level vertically?  Is the center of the hole

lined up with the center of the ODF 2000 inlet?

If not, line up.

4.  Are the front set of  hangers level vertically?

Do the rods hang straight down?

5.  Are the back set of hangers level vertically?

Do the rods hang straight down?

You may have to bolt the Unistrut to the bottom of

the bar joist using unistrut clamps. Then use the spring

nuts to secure the threaded rod.

6.  Install the 6” Nipple with the welded angle flange

such that the 2” section is pointing down through the

hood.

7.  Telescoping duct. Cut a piece of 6” pipe 8” long.

Insert into the nipple and clamp.

8.  All 16 drops MUST be hard piped.

9.  Install the butterfly dampers only on the outer pickups.

Leave fully open (we can adjust airflow if needed).

10.  Install P-Trap kit on the drain outlet.

Absolent is hung at correct height and is level ___________

(Please initial before we proceed with ductwork)

Ductwork is installed to all 4 drops _________

(Please initial with approval of installation)